SUPPORTING DATA

Atmospheric noble gases as tracers of biogenic gas dynamics in a shallow unconfined aquifer

Katherine L. Jones a,1, Matthew B.J. Lindsay b, Rolf Kipfer c,d, K. Ulrich Mayer a,*

a	Department of Earth, Ocean and Atmospheric Sciences, University of British Columbia, Vancouver, BC V6T 1Z4, Canada

b Department of Geological Sciences, University of Saskatchewan, Saskatoon, SK S7N 5E2, Canada

c Department of Water Resources and Drinking Water, Swiss Federal Institute of Aquatic Science and Technology (Eawag), CH-8600 Dübendorf, Switzerland

d	Institute for Geochemistry and Petrology, Swiss Federal Institute of Technology (ETH) Zürich, CH-8092 Zürich, Switzerland

* Corresponding author. Tel.: +1 604 822 1539
E-mail address: umayer@eos.ubc.ca (K.U. Mayer)

1	Present Address: BGC Engineering Inc, Vancouver, BC V6Z 2A9, Canada

- INPUT PARAMETERS FOR REACTIVE TRANSPORT MODELING OF VADOSE ZONE GAS DYNAMICS

The reactive transport model to describe noble gas advection and diffusion was built on previous model presented by Molins and Mayer (2007), Molins et al. (2010) and Sihota and Mayer (2012). In the following the input parameters used in the model are described.

The reactive transport model is based on 9 components, namely dissolved O2, N2, CO2, CH4, He, Ne, Ar, Kr, and Xe. Gases can be present both in dissolved form, as well as in gaseous form. CH4 and CO2 are assumed to be produced at the base of the domain constrained by a measured surficial gas efflux of 2.6 µmol m-2 s-1 (Sihota et al. 2011). Consistent with previous work by Molins and Mayer (2007), it was assumed that 85% of gas generated was in the form of CH4 and 15% in the form of CO2. The oxidation of CH4 by atmospheric oxygen in the methanotrophic zone was described by the reaction:

CH4(g)+2O2(g)→CO2)g)+2H2O

and subject to dual Monod rate expression of the form:

R=k[CH4]CH4+KCH4[O2]O2+KO2

where k is a calibrated rate coefficient (k = 1 x 10-6 mol L-1 H2O s-1. KCH4 and KCO2 are the half saturation constants for CH4 and O2 with values of 1.0 x 10-5 mol L-1 and 3 x 10-6 mol L-1, respectively.

The solution domain extended over 6.6 m from the ground surface to 0.7 m above the water table and was discretized into 40 cells. The simulation was carried out over a time period of one year to achieve steady state conditions.

The calibrated hydrogeological parameters are detailed in Table S.1. Parameters were kept in the range to those reported by Dillard et al. (1997) as previously used by Molins and Mayer (2007) and Sihota and Mayer (2012). A recharge flux of 180 mm yr-1 was specified as a flow boundary condition with a free outflow at the base.

Table S1: Key physical parameters for vadose zone reactive transport simulations

|  | Basal sand layer | Silt layer | Upper sand layer |
| --- | --- | --- | --- |
| Extent - elevation [m] | 425.2 – 426.7 | 426.7 – 427.6 | 427.6-431.8 |
| Hydraulic conductivity [m s-1] | 3 x 10-5 | 3 x 10-6 | 5.0 x 10-4 |
| Porosity | 0.33 | 0.33 | 0.4 |

Soil hydraulic function parameters were calibrated to match the observed gas composition achieving gas contents ranging from 0.16 at the base of the domain to 0.3 near the ground surface, consistent with previous studies (Dillard et al., 1997, Molins and Mayer, 2007, Sihota and Mayer, 2012).

The composition of the reactive gases and noble gases at the ground surface was fixed at atmospheric values using a first type Dirichlet boundary condition. The basal boundary was assumed to be impermeable. Atmospheric conditions were also assumed for the initial condition within the domain; however, this condition is arbitrary, as the simulations were run to steady state. Binary diffusion coefficients and gas solubilities used in the simulations are listed in Tables S2 and S3. On the method for determining binary diffusion coefficints, please refer to Molins et al (2010) for details.

Table S2: Binary diffusion coefficients for vadose zone reactive transport simulations (T = 8 oC).

| Binary mixture | Diffusion coefficient [m2 s-1] | Binary mixture | Diffusion coefficient [m2 s-1] |
| --- | --- | --- | --- |
| O2-CH4 | 2.01x10-5 | CO2-Xe | 9.3 x10-5 |
| O2-CO2 | 1.36 x10-5 | CO2-Kr | 7.43 x10-5 |
| O2-He | 6.81 x10-5 | CO2-N2 | 1.39 x10-5 |
| O2-Ne | 2.99 x10-5 | He-Ne | 9.71 x10-5 |
| O2-Ar | 1.77 x10-5 | He-Ar | 6.64 x10-5 |
| O2-Xe | 1.36 x10-5 | He-Xe | 5.9 x10-5 |
| O2-Kr | 1.11 x10-5 | He-Kr | 5.05 x10-5 |
| O2-N2 | 1.86 x10-5 | He-N2 | 6.38 x10-5 |
| CH4-CO2 | 1.50 x10-5 | Ne-Ar | 2.85 x10-5 |
| CH4-He | 6.34 x10-5 | Ne-Xe | 2.32 x10-5 |
| CH4-Ne | 3.1 x10-5 | Ne-Kr | 1.93 x10-5 |
| CH4-Ar | 1.94 x10-5 | Ne-N2 | 2.89 x10-5 |
| CH4-Xe | 1.55 x10-5 | Ar-Xe | 1.26 x10-5 |
| CH4-Kr | 1.29 x10-5 | Ar-Kr | 1.02 x10-5 |
| CH4-N2 | 2.00 x10-5 | Ar-N2 | 1.77 x10-5 |
| CO2-He | 5.43 x10-5 | Kr-Xe | 6.75 x10-5 |
| CO2-Ne | 2.26 x10-5 | Kr-N2 | 1.39 x10-5 |
| CO2-Ar | 1.28 x10-5 | Xe-N2 | 1.15 x10-5 |

Table S3: Henry’s law constants for vadose zone reactive transport simulations, also used for degassing and ebullition simulations (T = 8oC).

| Gas | Henry’s law constant [mol L-1 atm-1] reported as log KH,cp |
| --- | --- |
| O2 | -2.77 |
| CH4 | -2.86 |
| CO2 | -1.47 |
| N2 | -3.19 |
| He | -3.40 |
| Ne | -3.30 |
| Ar | -2.72 |
| Kr | -2.44 |
| Xe | -2.17 |

- DESCRIPTION OF PHRREQCI MODEL FOR GAS EXSOLUTION DEGASSING, AND EBULLITION

PHREEQCi (v3.06.7757) (Parkhurst and Appelo, 2013) was utilized to simulate either i) gas exsolution and bubble formation or ii) gas exsolution, bubble formation and periodic gas bubble release via ebullition.

- Gas exsolution and bubble formation

An initial solution was specified for Air Saturated Water (ASW), but with oxygen removed and replaced by CH4 and CO2 at partial pressures of 0.105atm. The GAS-PHASE keyword was utilized to simulate bubble formation under constant pressure conditions (p = 0.95 atm). Gas bubble formation was induced by the addition of CH4 and CO2 in equal parts, specified using the REACTION keyword. At each reaction step 10-6 mol/LH2O was added for each gas to mimic gas generation by biodegradation. The gas phase was allowed to grow, and gas was not removed from the system. Equilibrium between the gas phase and the aqueous phase was maintained continuously. The Henry’s constants reports in Table S3 were used for this simulation.

- Gas exsolution and bubble formation

This model is based an extension of model i), with the only difference that the exsolved gas phase is removed immediately after completion of each reaction step to mimic the process of ebullition. The water composition is saved for the next reaction step. Further addition of CH4 and CO2 via the REACTION keyword leads to the nucleation of a new gas phase. The noble gases remaining in the water are then equilibrated between this new gas phase and the water.

# References

Dillard L. A., Essaid H. I. and Herkelrath W. N. (1997) Multiphase flow modeling of a crude-oil spill site with a bimodal permeability distribution. Water Resour. Res. 33, 1617–1632.

Molins S. and Mayer K. U. (2007). Coupling between geochemical reactions and multicomponent gas and solute transport in unsaturated media: A reactive transport modeling study. Water Resour. Res. 43, W05435.

Molins S., Mayer K. U., Amos R. T. and Bekins B. A. (2010) Vadose zone attenuation of organic compounds at a crude oil spill site – Interactions between biogeochemical reactions and multicomponent gas transport. J. Contam. Hydrol. 112, 15–29.

Parkhurst, D.L., and Appelo, C.A.J., 2013, Description of input and examples for PHREEQC version 3—A computer program for speciation, batch-reaction, one-dimensional transport, and inverse geochemical calculations: U.S. Geological Survey Techniques and Methods, book 6, chap. A43, 497 p., available only at http://pubs.usgs.gov/tm/06/a43.

Sihota N. J., Singurindy O. and Mayer K. U. (2011) CO2-Efflux measurements for evaluating source zone natural attenuation rates in a petroleum hydrocarbon contaminated aquifer. Environ. Sci. Technol. 45, 482–488.

Sihota, N.J., and K.U. Mayer, 2012. Characterizing vadose zone hydrocarbon biodegradation using CO2-effluxes, isotopes, and reactive transport modeling, Vadose Zone J., 11, doi:10.2136/vzj2011.0204.