# Implementation of Noble Gas Ingrowth

# To do list

# Literature review of noble gas ingrowth;

# Add different noble gas into database;

# Consider noble gas absorption and partition in organic materials such as oil.

# 1. Ingrowth of helium

## 1.1 Review of Ballentine and Burnard 2002, Production, Release and Transport of Noble Gases in the Continental Crust.

Looks like Equation 12 is not quite right. The rate should be divided by time t.

4He atoms g-1 yr-1=XrRNA/Ar×10-6eλt-1×yieldr/t

Or

4He atoms g-1 yr-1=XrRNA/Ar×10-6eλ-1×yieldr

Otherwise, put different t in equation 12 will have quite different production rate. Verification is shown in the table below,  which matches equation 13 if the modified formula is used.

We can either the above modified one (more general, more parameters) or equation 13 (based on natural parameters, less parameters, but may not be used as specific case).

| Variables | U238 | U235 | Th232 |
| --- | --- | --- | --- |
| Xr | 0.9928 | 0.0072 | 1 |
| [R] | 1 | 1 | 1 |
| Na | 6.02E+23 | 6.02E+23 | 6.02E+23 |
| Ar | 238.051 | 235.044 | 232.038 |
| yield | 8 | 7 | 6 |
| lamda | 1.55E-10 | 9.85E-10 | 4.95E-11 |
| t | 2 | 2 | 2 |
| Rate [g-1 yr-1] - eq 12 | 6.230E+06 | 2.544E+05 | 1.542E+06 |
| Rate [g-1 yr-1] - new | 3.115E+06 | 1.272E+05 | 7.709E+05 |

The results in blue is calculated based on equation 12, which does not match equation 13 if t is not 1.

Can the production rate of 3He considered constant for the upper curst?

The earth’s crust contains an average of about 2-4 ppmuranium, 8.1 ppm thorium.

Average production rate of 4He calculated based on the correction equation is 1.27e+7 to 1.92e+7, which is consistent with the value 1.73e+7 in Table 3

| Variable | Group 1 |  |  |  | Group 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | U238 | U235 | Th232 |  | U238 | U235 | Th232 |
| Xr | 0.9928 | 0.0072 | 1 |  | 0.9928 | 0.0072 | 1 |
| [R] | 2.00E+00 | 2.00E+00 | 8.1 |  | 4.00E+00 | 4.00E+00 | 8.1 |
| Na | 6.02E+23 | 6.02E+23 | 6.02E+23 |  | 6.02E+23 | 6.02E+23 | 6.02E+23 |
| Ar | 238.051 | 235.044 | 232.038 |  | 238.051 | 235.044 | 232.038 |
| yield | 8 | 7 | 6 |  | 8 | 7 | 6 |
| lamda | 1.55E-10 | 9.85E-10 | 4.95E-11 |  | 1.55E-10 | 9.85E-10 | 4.95E-11 |
| t | 2 | 2 | 2 |  | 2 | 2 | 2 |
| Rate [g-1 yr-1] | 6.230E+06 | 2.544E+05 | 6.244E+06 |  | 1.246E+07 | 5.089E+05 | 6.244E+06 |
| Sum of rate | 1.273E+07 |  |  |  | 1.921E+07 |  |  |

## 1.2 Rate production formula to be implemented

For equation 13, the required parameters include the concentration of [U] and [Th] in solid phase, ppm or g/m^3 solid?

For more general equation, we need fractional natural abundance of isotope R, Avogadro’s number Na (constant), Molar mass of R (g, also constant), decay constant of R (yr-1, constant), number of alpha particles emitted in the complete decay chain (8, 7, 6 for U238, U235 and Th232).

4He atoms g-1 yr-1=XrRNA/Ar×10-6eλt-1×yieldr/t

The production of He (mol) during one time step for one control volume can be calculated by

Q = 4He (atoms  g-1 yr-1) *delt (yr) * cvol (m3) * bulk density (g m-3) / 6.02e23  (atmos mol-1) => x mol

Do we need to consider 3He? If considered, use user specified rate of 3He or user specified ratio of  3He/4He?

## 1.3 How to implement in MIN3P code

Taking Helium as an example.

- Aqueous components be added (4He(aq)).

- Gas species to be added (4He).

- Reaction between aqueous and gas phases: 4He(aq)  4He, 3He(aq)  3He.

- Reaction between aqueous and mineral phases to represent production of helium and then calculate the updated aqueous concentration and gas saturation. This part is implemented as a source/sink term. We calculate how much He is generated during current time step delt and pass it to the right hand side. Since the production rate does not dependent on the aqueous phase of [U] and [Th], no Jacobi matrix modification is required.

- Verification examples? Compare results to the model without considering noble gas ingrowth. What kind of conceptual model shall we use?

I’d like to get this done in more general terms, as follows:

Ideally this would be done through mineral.dbs with a series of first-order decay reactions and the opportunity to specify the initial 238U, 235U and 232U concentrations in the domain, as a function of rock unit. This will make use of the existing framework for source-sink terms and can easily be expanded. However, this will require some modifications, because:

- The release of noble gases is dependent on a solid phase concentration, not an aqueous concentration. mineral.dbs does not allow to express a first order reaction based on a solid phase concentration at this time. This would have to be added.

- It would be nice to keep track of the products (not only He). This is also not facilitated by mineral.dbs at this point, since interactions only take place between a single mineral and aqueous components. This would require to include consumption and production of a solid phase. This step is probably more complicated, but much less important. Probably best to leave out for now.

This can be done as a function of volume fraction directly or solid phase concentration. In the future, this would lend itself well to extend the formulation to look at noble gas ingrowth directly linked to mineralogical composition.  For example, the reaction 238U -> 206Pb + 8 4He would go in the database along with the first order decay constant. 238U and 206Pb would be solids. 4He is the aqueous component. The rate would be written as R = - k [238U], leading to the production of 206Pb  (no need to keep track of that initially) and the release of 8 4He to the water phase. Since there is no limit on 238U over time scales of interest, this should be fairly straightforward.      238U (and possibly 206Pb) can be updated at end of time step. Concentrations would have to be given in terms of [mol L-1 bulk] (volume fraction does not make sense in this case). Therefore some adjustments needed re input in .dat file, or .min file.

We can discuss this in a meeting next week.

Specifically for the case with He (from Wikipedia https://en.wikipedia.org/wiki/Uranium-238):

Note all rate coefficients end up the same as in Ballantine paper.

238U – yield 8 He

Effectively (because 1st step is rate-determining): 238U -> 206Pb + 8 4He    with k = 0.693/ t1/2 = 0.693 / 4.468 x 10^9 = 1.55 x 10-10 yr-1

R = - k [238U] or in terms of production of 4He: R = 8 k [238U], note that 238U is U in solid phase

235U – yield 7 He

Effectively: 235U -> 207Pb + 7 4He    with k = 0.693/ t1/2 = 0.693 / 7.038 x 10^8 = 9.85 x 10-10 yr-1

R = - k [235U] or in terms of production of 4He: R = 7 k [235U], note that 235U is U in solid phase

232 Th – Yield 6 He

Effectively: 232Th -> 208Pb + 6 4He    with k = 0.693/ t1/2 = 0.693 / 1.4 x 10^10 = 4.95 x 10-11 yr-1

R = - k [232Th] or in terms of production of 4He: R = 6 k [232Th], note that 232Th is Th in solid phase

### Formulation and keywords to be used

Rate formulation: R = - k [M] =ln2/(t_halflife)

### Input file

‘Components’

‘4He(aq)’

‘206Pb(aq)’      ;can be ignored if this is not necessary to be considered

‘207Pb(aq)’

‘208Pb(aq)’

‘minerals’

‘238U-yield-8He’

‘235U-yield-7He’

‘232Th-yield-6He’

‘scaling for  kinetic reactions’

4.919e-18   ; s-1, = 1.55 x 10-10 yr-1

3.123e-17   ; s-1, = 9.85 x 10-10 yr-1

1.570e-18   ; s-1, = 4.95 x 10-11 yr-1

### Database mineral.dbs

! Noble gas ingrowth - May 2022

!

!'238u-yield-8he'

!

'238u-yield-8he'

'surface'

238.0    19.1                     ;to be updated

1  '4he(aq)'  8                   ; 2  '206Pb(aq)'  1.0  '4he(aq)'  8

'irreversible dissolution'

'log rate constant' -17.308

'fractional C^m'    1             ;function of solid or mineral phase concentration

'238u-yield-8he'    1

!

!'235u-yield-7he'

!

'235u-yield-7he'

'surface'

235.0  19.1                       ;to be updated

1  '4he(aq)'  7                   ;  2  '207Pb(aq)'  1.0  '4he(aq)'  7

'irreversible dissolution'

'log rate constant' -16.505

'fractional C^m'    1             ;function of solid or mineral phase concentration

'235u-yield-7he'    1

!

!'232th-yield-6he'

!

'232th-yield-6he'

'surface'

232.0  11.7                       ;to be updated

1  '4he(aq)'  6                   ; 2  '208Pb(aq)'  1.0  '4he(aq)'  6

'irreversible dissolution'

'log rate constant' -17.804

'fractional C^m     1             ;function of solid or mineral phase concentration

'232th-yield-6he'   1

Batch reaction; 1D diffusion; probably DGR site?

Assuming [238U] = 1ppm = 1mg L-1 = 1g/m3 solid, cvol = 1 m3, delt = 1 year,

1 g 238U equals 106 g rock, generates 4He in one year according to equation 13

Q (4He)= 3.115e6*1*1*1*1.0e6/6.02e23=5.17e-12 mol

1 g 238U generates 4He in one year according to rate constant from wikipedia

238U -> 206Pb + 8 4He    with k = 0.693/ t1/2 = 0.693 / 4.468 x 10^9 = 1.55 x 10-10 yr-1

Q(4He)=1.0/238*1.55e-10*8=5.21e-12 mol

Andrews et al. 1989, Atmospheric and radiogenic gases in groundwaters from the Stripa granite, Geochimica et Cosmochimica Acta,  Vol 53, pp. 1831-1841

Static fluid in a fracture within a rock containing stored He must gradually acquire He by diffusion out of the rock matrix. An equilibrium is established when the chemical potentials for He in the rock matrix and in the water phase are equal.